# Supplementary Materials

Receiver-dependent precise-product sensitivity and raw-observation quality in PPP

# Table S1. Receiver-specific ITRF2020 ECEF Ground-Truth coordinates.

| Test | Receiver | X (m) | Y (m) | Z (m) |
| --- | --- | --- | --- | --- |
| Test-1 | T11 | 4213629.71 | 2370585.44 | 4146228.73 |
| Test-1 | OEM1 | 4213629.53 | 2370585.85 | 4146228.62 |
| Test-1 | C1 | 4213630.34 | 2370585.56 | 4146228.14 |
| Test-1 | T21 | 4213630.14 | 2370585.95 | 4146228.01 |
| Test-2 | T12 | 4213624.43 | 2370584.55 | 4146234.59 |
| Test-2 | OEM2 | 4213624.23 | 2370584.96 | 4146234.50 |
| Test-2 | C2 | 4213625.04 | 2370584.73 | 4146233.98 |
| Test-2 | T22 | 4213624.81 | 2370585.13 | 4146233.88 |
| Test-3 | T13 | 4213629.45 | 2370578.62 | 4146232.87 |
| Test-3 | OEM3 | 4213629.25 | 2370579.03 | 4146232.78 |
| Test-3 | C3 | 4213630.06 | 2370578.78 | 4146232.27 |
| Test-3 | T23 | 4213629.84 | 2370579.17 | 4146232.16 |

Note. Coordinates are the receiver-specific Ground Truth used in the final PPP error analysis. Values are rounded to 0.01 m for presentation; computations used the full-precision source values.

# Table S2. Stage-1 raw-observation quality metrics.

## Table S2a. Signal level and code-multipath metrics.

| Test | Receiver | Class | C/N₀ median (dB-Hz) | C/N₀ robust σ (dB) | L1 CMC robust σ (m) | L1 CMC P95¦·¦ (m) |
| --- | --- | --- | --- | --- | --- | --- |
| Test-1 | C1 | Geodetic | 44.00 | 4.45 | 2.10 | 6.84 |
| Test-1 | OEM1 | OEM | 41.00 | 5.93 | 0.21 | 1.61 |
| Test-1 | T11 | Smartphone | 34.10 | 5.78 | 16.01 | 32.52 |
| Test-1 | T21 | Smartphone | 33.76 | 5.68 | 9.13 | 19.95 |
| Test-2 | C2 | Geodetic | 44.00 | 4.45 | 0.92 | 5.00 |
| Test-2 | OEM2 | OEM | 41.00 | 4.45 | 1.63 | 7.57 |
| Test-2 | T12 | Smartphone | 33.27 | 6.33 | 26.55 | 52.58 |
| Test-2 | T22 | Smartphone | 32.49 | 6.00 | 17.38 | 36.78 |
| Test-3 | C3 | Geodetic | 44.00 | 4.45 | 0.93 | 5.06 |
| Test-3 | OEM3 | OEM | 41.00 | 5.93 | 2.07 | 8.70 |
| Test-3 | T13 | Smartphone | 33.00 | 6.52 | 20.27 | 41.81 |
| Test-3 | T23 | Smartphone | 32.83 | 6.12 | 11.20 | 22.74 |

## Table S2b. Continuity, retention, completeness, and signal-availability metrics.

| Test | Receiver | Phase disc. (/1000) | Median arc (s) | Epoch retention (%) | Obs. completeness (%) | Usable sats. (median) | Multi-freq. (%) | Signal diversity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test-1 | C1 | 1.29 | 14.00 | 100.00 | 100.00 | 30.00 | 77.99 | 9 |
| Test-1 | OEM1 | 0.96 | 47.00 | 14.59 | 68.55 | 37.00 | 59.64 | 9 |
| Test-1 | T11 | 0.24 | 465.00 | 99.77 | 83.79 | 45.00 | 34.63 | 6 |
| Test-1 | T21 | 0.62 | 298.00 | 99.83 | 83.94 | 46.00 | 34.52 | 6 |
| Test-2 | C2 | 1.24 | 13.00 | 100.00 | 100.00 | 32.00 | 84.55 | 9 |
| Test-2 | OEM2 | 0.14 | 384.50 | 100.00 | 70.35 | 36.00 | 61.29 | 9 |
| Test-2 | T12 | 0.17 | 216.00 | 100.00 | 85.42 | 50.00 | 36.75 | 6 |
| Test-2 | T22 | 0.25 | 204.00 | 99.76 | 85.91 | 50.00 | 37.10 | 6 |
| Test-3 | C3 | 0.68 | 120.00 | 100.00 | 100.00 | 32.00 | 90.03 | 9 |
| Test-3 | OEM3 | 0.52 | 69.00 | 100.00 | 74.25 | 36.00 | 65.47 | 9 |
| Test-3 | T13 | 0.97 | 117.00 | 99.84 | 81.73 | 47.00 | 30.96 | 6 |
| Test-3 | T23 | 0.39 | 135.00 | 99.89 | 82.21 | 46.00 | 31.11 | 6 |

Note. Table S2 contains the manuscript-facing Stage-1 metrics only. File paths and internal processing identifiers are intentionally omitted.

# Table S3. Complete Stage-2 PPP accuracy across the six precise-product configurations.

## Table S3a. CLEAN horizontal RMSE (m).

| Test | Receiver | CODE | GFZ | GRG | IGS | JAX | WHU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test-1 | C1 | 0.07 | 0.15 | 0.08 | 0.25 | 0.61 | 0.35 |
| Test-1 | OEM1 | NR | NR | NR | NR | NR | NR |
| Test-1 | T11 | 2.72 | 4.33 | 5.62 | 5.71 | 8.84 | 2.80 |
| Test-1 | T21 | 2.78 | 5.01 | 6.51 | 6.66 | 7.82 | 2.79 |
| Test-2 | C2 | 0.25 | 0.84 | 0.08 | 0.06 | 0.22 | 0.30 |
| Test-2 | OEM2 | 0.54 | 0.18 | 0.06 | 4.42† | 0.49 | NR |
| Test-2 | T12 | 5.68 | 9.06 | 7.37 | 7.44 | 6.73 | 5.47 |
| Test-2 | T22 | 4.08 | 9.41 | 6.23 | 6.28 | 6.79 | 4.17 |
| Test-3 | C3 | 0.34 | 1.02 | 0.13 | 0.33 | 0.22 | 0.32 |
| Test-3 | OEM3 | 1.33 | 0.31 | 0.03 | 0.87 | 0.08 | 46.69† |
| Test-3 | T13 | 2.15 | 4.63 | 3.40 | 3.40 | 3.51 | 1.83 |
| Test-3 | T23 | 0.48 | 2.90 | 1.45 | 1.43 | 1.76 | 0.53 |

## Table S3b. CLEAN three-dimensional RMSE (m).

| Test | Receiver | CODE | GFZ | GRG | IGS | JAX | WHU |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test-1 | C1 | 0.27 | 0.43 | 0.28 | 0.44 | 2.59 | 0.51 |
| Test-1 | OEM1 | NR | NR | NR | NR | NR | NR |
| Test-1 | T11 | 2.90 | 4.80 | 6.33 | 6.44 | 9.40 | 2.82 |
| Test-1 | T21 | 2.85 | 5.53 | 6.55 | 6.70 | 8.05 | 3.20 |
| Test-2 | C2 | 0.26 | 0.87 | 0.15 | 0.11 | 0.65 | 0.32 |
| Test-2 | OEM2 | 0.54 | 0.58 | 0.36 | 5.03† | 0.86 | NR |
| Test-2 | T12 | 6.05 | 9.44 | 7.42 | 7.49 | 11.91 | 6.65 |
| Test-2 | T22 | 5.00 | 10.58 | 6.29 | 6.36 | 7.72 | 7.64 |
| Test-3 | C3 | 0.34 | 1.13 | 0.13 | 0.33 | 0.22 | 0.33 |
| Test-3 | OEM3 | 1.41 | 0.36 | 0.10 | 1.21 | 0.35 | 63.18† |
| Test-3 | T13 | 2.35 | 5.05 | 3.58 | 3.57 | 3.68 | 2.92 |
| Test-3 | T23 | 4.61 | 8.31 | 3.93 | 4.08 | 4.74 | 5.85 |

Note. NR = no Stage-2 result row; † = processing result flagged FAILED because of insufficient CLEAN epochs. Failed solutions are shown for traceability and are not treated as usable solutions.

# Table S4. Precise-product and navigation provenance.

| Center | Orbit file (SP3) | Clock file (CLK) | Center OSB file (BIA) |
| --- | --- | --- | --- |
| CODE | COD0MGXFIN_20231560000_01D_05M_ORB.SP3 | COD0MGXFIN_20231560000_01D_30S_CLK.CLK | COD0MGXFIN_20231560000_01D_01D_OSB.BIA |
| GFZ | GFZ0MGXRAP_20231560000_01D_05M_ORB.SP3 | GFZ0MGXRAP_20231560000_01D_30S_CLK.CLK | GFZ0MGXRAP_20231560000_01D_01D_OSB.BIA |
| GRG | GRG0MGXFIN_20231560000_01D_05M_ORB.SP3 | GRG0MGXFIN_20231560000_01D_30S_CLK.CLK | GRG0MGXFIN_20231560000_01D_01D_OSB.BIA |
| IGS | IGS0OPSFIN_20231560000_01D_15M_ORB.SP3 | IGS0OPSFIN_20231560000_01D_30S_CLK.CLK | — |
| JAX | JAX0MGXFIN_20231560000_01D_05M_ORB.SP3 | JAX0MGXFIN_20231560000_01D_30S_CLK.CLK | — |
| WHU | WUM0MGXRAP_20231560000_01D_05M_ORB.SP3 | WUM0MGXRAP_20231560000_01D_30S_CLK.CLK | WUM0MGXRAP_20231560000_01D_01D_OSB.BIA |

Note. All three Tests were acquired on the same observation date; repeated per-Test filename rows are therefore collapsed. Broadcast multi-GNSS navigation: BRDM00DLR_S_20231560000_01D_MN.rnx. Cross-center OSB use, where applicable, follows the operational configurations defined in main-text Table 2.

# Table S5. Primary and sensitivity association results.

| Outcome layer | Predictor | N | Spearman ρ | Exact p | Permutations | LOTO ρ range |
| --- | --- | --- | --- | --- | --- | --- |
| Product-adjusted | CMC P95 | 11 | 0.818 | 0.0069 | 3456 | 0.643 to 0.964 |
| Product-adjusted | CMC robust σ | 11 | 0.836 | 0.0049 | 3456 | 0.714 to 0.929 |
| Product-adjusted | C/N₀ | 11 | -0.745 | 0.0171 | 3456 | -0.955 to -0.559 |
| Unadjusted | CMC P95 | 11 | 0.845 | 0.0035 | 3456 | 0.643 to 0.929 |
| Unadjusted | CMC robust σ | 11 | 0.791 | 0.0049 | 3456 | 0.607 to 0.905 |
| Unadjusted | C/N₀ | 11 | -0.791 | 0.0049 | 3456 | -0.955 to -0.613 |

Note. The confirmatory outcome is the product-adjusted median CLEAN horizontal RMSE. Unadjusted rows are retained as sensitivity checks. LOTO = leave-one-Test-out.

# Table S6. Reviewer-driven reproducibility and sensitivity audits.

## Table S6a. Dataset-level sampling audit.

| Test | Receiver | Epochs | Expected | Median Δt (s) | Nominal (s) | Retention (%) |
| --- | --- | --- | --- | --- | --- | --- |
| Test-1 | C1 | 5601.0 | 5601.0 | 1.00 | 1 | 100.00 |
| Test-1 | OEM1 | 966.0 | 6620.0 | 1.00 | 1 | 14.59 |
| Test-1 | T11 | 5252.0 | 5264.0 | 1.00 | 1 | 99.77 |
| Test-1 | T21 | 5271.0 | 5280.0 | 1.00 | 1 | 99.83 |
| Test-2 | C2 | 5193.0 | 5193.0 | 1.00 | 1 | 100.00 |
| Test-2 | OEM2 | 3566.0 | 3566.0 | 1.00 | 1 | 100.00 |
| Test-2 | T12 | 3723.0 | 3723.0 | 1.00 | 1 | 100.00 |
| Test-2 | T22 | 3710.0 | 3719.0 | 1.00 | 1 | 99.76 |
| Test-3 | C3 | 3660.0 | 3660.0 | 1.00 | 1 | 100.00 |
| Test-3 | OEM3 | 3186.0 | 3186.0 | 1.00 | 1 | 100.00 |
| Test-3 | T13 | 3654.0 | 3660.0 | 1.00 | 1 | 99.84 |
| Test-3 | T23 | 3651.0 | 3655.0 | 1.00 | 1 | 99.89 |

## Table S6b. CMC-treatment sensitivity for the primary horizontal outcome.

| CMC treatment | CMC statistic | N | Spearman ρ | p |
| --- | --- | --- | --- | --- |
| Current | Robust σ | 11 | 0.791 | 0.0037 |
| Current | P95 ¦CMC¦ | 11 | 0.845 | 0.0010 |
| Fixed 300 s | Robust σ | 11 | 0.718 | 0.0128 |
| Fixed 300 s | P95 ¦CMC¦ | 11 | 0.700 | 0.0165 |
| Linear | Robust σ | 11 | 0.645 | 0.0320 |
| Linear | P95 ¦CMC¦ | 11 | 0.655 | 0.0289 |
| Quadratic | Robust σ | 11 | 0.700 | 0.0165 |
| Quadratic | P95 ¦CMC¦ | 11 | 0.591 | 0.0556 |

Note. The audit table is restricted to the primary all-dataset horizontal outcome to avoid duplicating secondary and subgroup analyses. Full machine-readable audit exports remain the provenance source.